| N427AB | 2980 AIRWAY AVE, COSTA MESA, CA 92626 TEL: 714-433-2275, WWW.JAYSAIRCRAFT.COM FAA APPROVED REPAIR STATION # JVYR016L |  | DATE: 8/26/219 |
| --- | --- | --- | --- |
| S/N: 4636324 |  |  | W.O.#: 22828 |
| MODEL: PA46-350P |  |  | TSMOH: |
| TACH TIME: |  |  | TOTAL TIME: 2108.9 |
| HOBBS TIME: 2108.9 |  |  |  |
| AIRFRAME LOG THE AIRCRAFT AND / OR COMPONENT IDENTIFIED ABOVE WAS REPAIRED AND INSPECTED IN ACCORDANCE WITH CURRENT FEDERAL AVIATION REGULATIONS AND WAS FOUND AIRWORTHY FOR RETURN TO SERVICE. PERTINENT DETAILS OF THE REPAIR / REPLACEMENT ARE ON FILE AT THIS AGENCY SIGNED: _____________________________________________________________________________ |  |  |  |
| C/W ANNUAL INSPECTION IAW PIPER PA46-350P MAINTENANCE MANUAL. |  |  |  |
| FAR 91-207(D) COMPLY WITH YEARLY INSPECTION OF ELT BATTERY BY PHYSICAL TEST AND VISUAL INSPECTION. OPERATION CHECKED SATISFACTORY. R&R ELT BATTERY WITH NEW P/N 452-0130, S/N 382790-024. BATTERY EXPIRES 3/2022 |  |  |  |
| 24 MONTH PITOT/STATIC/TRANSPONDER CERTIFICATION PERFORMED IAW WITH FAR 91.411 AND FAR 91.413 BY OUTSIDE VENDOR UNDER INVOICE# 14121. NEXT DUE 8/2021 |  |  |  |
| VACUUM SYSTEM INSPECTED IAW PIPER PA46-350P SERVICE MANUAL IAW ANNUAL INSPECTION GUIDELINES. R&R VACUUM REGULATOR FILTERS WITH NEW P/N B3-5-1 (2). R&R CENTRAL INSTRUMENT FILTER WITH NEW P/N AM103535IA. R&R VACUUM PUMP FILTER WITH NEW P/N D9-18-1. OPERATIONAL CHECK FOUND SATISFACTORY. |  |  |  |
| R&R MAIN BATTERY WITH NEW GILL P/N G242 WITH ELECTROLYTE IAW PIPER PA46-350P SERVICE MANUAL. OPERATIONAL CHECK FOUND SATISFACTORY. |  |  |  |
| TROUBLESHOT AIR CONDITIONING SYSTEM IAW PIPER PA46-350P SERVICE MANUAL. FOUND SYSTEM REFRIGERANT EMPTY. VACUUMED SYSTEM AND SERVICED WITH R134A FREON AS REQUIRED. OPERATIONAL AND LEAK CHECK FOUND SATISFACTORY. |  |  |  |
| R&R L/H BRAKE CALIPER AFTER REPLACING O-RINGS WITH NEW P/N MS28775-222 (2) IAW PIPER AP46-350P AND CLEVELAND BRAKE SERVICE MANUALS SERVICED AND BLED SYSTEM USING MIL-5606-H HYDRAULIC FLUID. OPERATIONAL AND LEAK CHECK FOUND SATISFACTORY. |  |  |  |
| R&R L/H BRAKE LININGS WITH NEW P/N 066-62 (4) AND BRAKE SHIM P/N 68-01100 IAW PIPER PA46-350P AND CLEVELAND BRAKE SERVICE MANUALS. OPERATIONAL AND LEAK CHECK FOUND SATISFACTORY. |  |  |  |
| SOME SBS THAT APPLY TO THIS AIRCRAFT MAY NOT HAVE BEEN PERFORMED PER OWNER'S DECISION. C/W ALL APPLICABLE ADS THROUGHOUT THIS DATE. |  |  |  |