| N427AB | 2980 AIRWAY AVE, COSTA MESA, CA 92626 TEL: 714-433-2275, WWW.JAYSAIRCRAFT.COM FAA APPROVED REPAIR STATION # JVYR016L |  | DATE: 7/19/2018 |
| --- | --- | --- | --- |
| S/N: L-11015-61A |  |  | W.O.#: 22799 |
| MODEL: TIO-540-AE2A |  |  | TSMOH: |
| TACH TIME: |  |  | TOTAL TIME: |
| HOBBS TIME: 2108.9 |  |  |  |
| ENGINE LOG THE AIRCRAFT AND / OR COMPONENT IDENTIFIED ABOVE WAS REPAIRED AND INSPECTED IN ACCORDANCE WITH CURRENT FEDERAL AVIATION REGULATIONS AND WAS FOUND AIRWORTHY FOR RETURN TO SERVICE. PERTINENT DETAILS OF THE REPAIR / REPLACEMENT ARE ON FILE AT THIS AGENCY SIGNED: _____________________________________________________________________________ |  |  |  |
| R&R ALL SPARKS PLUGS AFTER CLEANED AND REPLACED GAKETS WITH NEW P/N M674 (12). |  |  |  |
| INSPECTED IGNITION SYSTEM AND FOUND BROKEN P-LEAD WIRE ON L/H MAGNETO. R&R L/H MAGNETO P-LEAD TERMINAL WITH NEW. OPERATIONAL AND SECURITY CHECK FOUND SATISFACTORY. |  |  |  |
| R&R OL FILTER WITH NEW P/N CH48103-1 IAW LYCOMING TIO-540 SERVICE MANUAL. FILTER CUT OPEN AND INSPECTED FOR METAL CONTAMINATION. SERVICED WITH LYCOMING ADDATIVE LW16702. FOUND SATISFACTORY. OPERATIONAL AND LEAK CHECK FOUND SATISFACTORY. |  |  |  |