| N427AB | 2980 AIRWAY AVE, COSTA MESA, CA 92626 TEL: 714-433-2275, WWW.JAYSAIRCRAFT.COM FAA APPROVED REPAIR STATION # JVYR016L |  | DATE: 8/26/219 |
| --- | --- | --- | --- |
| S/N: L-12384-61A |  |  | W.O.#: 22828 |
| MODEL: TIO-540-AE2A |  |  | TSMOH: |
| TACH TIME: |  |  | TOTAL TIME: 1476.2 |
| HOBBS TIME: 2108.9 |  |  |  |
| ENGINE LOG THE AIRCRAFT AND / OR COMPONENT IDENTIFIED ABOVE WAS REPAIRED AND INSPECTED IN ACCORDANCE WITH CURRENT FEDERAL AVIATION REGULATIONS AND WAS FOUND AIRWORTHY FOR RETURN TO SERVICE. PERTINENT DETAILS OF THE REPAIR / REPLACEMENT ARE ON FILE AT THIS AGENCY SIGNED: _____________________________________________________________________________ |  |  |  |
| C/W ANNUAL INSPECTION IAW PIPER PA46-350P MAINTENANCE MANUAL. |  |  |  |
| REMOVED ENGINE MODEL TIO-540-AE2A, S/N L-11015-61A AND REPLACED WITH SERVICEABLE ENGINE FROM PIPER PA46-350P, N333GC, S/N 4636383, ENGINE MODEL TIO-540-AE2A, S/N L-12384-61A. ENGINE INSTALLED WAS INSTALLED AS COMPLETE ASSEMBLY FROM FIREWALL FORWARD INCLUDING ENGINE MOUNTS, ALL ACCESSORIES, HOSES, AND EXHAUST SYSTEM. REMOVED R/H TURBO CHARGER FROM L-11015-61A ENGINE AND INSTALLED ON L-12384-61A ENGINE AFTER FLUSHING TURBOCHARGER OIL SUPPLY DUE TO ORIGINAL TURBOCHARGER LEAKING OIL. R&R ALL ENGINE MOUNT HARDWARE WITH NEW P/N 692-107, 692-106 (2), MS21044N7 (10), AN3-11 (5), AN6-32A (2). OPERATIONAL AND LEAK CHECK FOUND SATISFACTORY. AIRCRAFT GROUND RUN AND FOUND SATISFACTORY. SERVICED WITH 11 QTS OF AEROSHELL W100 AND CH48103-1 OIL FILTER. AOA TAKEN. ENGINE WASHED, RUN UP AND INSPECTED. OPERATIONAL CHECK SATISFACTORY. |  |  |  |
| COMPRESSION VALUES: 1. 70/80, 2. 72/80, 3. 70/80, 4. 74/80, 5. 70/80, 6. 72/80 MASTER ORIFICE VALUE: 42 |  |  |  |
| R&R INDUCTION AIR FILTER AFTER CLEANED AND INSPECTED. FOUND SATISFACTORY. |  |  |  |
| MAGNETOS INSPECTED IAW SLICK 6300 SERIES, PIPER PA46-350P, AND LYCOMING TIO-540 SERVICE MANUALS IAW ANNUAL INSPECTION GUIDELINES. RE-TIMED MAGNETOS AS REQUIRED. OPERATIONAL CHECK FOUND SATISFACTORY. 500 HOUR MAGNETO INSPECTION DUE @ 2518.7 HOBBS HOURS, (409.8 HOURS REMAINING) |  |  |  |
| SPARK PLUGS ROTATED AND CLEANED, REPLACED GASKETS WITH NEW, P/N M674 (12) |  |  |  |
| C/W AD2015-19-07 LYCOMING FUEL INJECTION LINE BY INSPECTION. FOUND SATISFACTORY. |  |  |  |
| C/W AD2011-06-01 BY REPLACEMENT OF TIT PROBE WITH NEW P/N 686-216 IAW PIPER PA46-350P AND LYCOMING TIO-540 SERVICE MANUALS. OPERATIONAL CHECK FOUND SATISFACTORY. NEXT DUE AT 2358.9 HOBBS TIME |  |  |  |
| ALL CHT, EGT, MP, RPM, AND OIL TEMP PROBES NOT COMPATIBLE WITH CURRENT AIRFRAME SYSTEM R&R ALL CHT AND EGT PROBES WITH ADAPTERS P/N 28202. CUT AND SPLICED ALL WIRING TO NON COMPATIBLE SENSORS AS REQUIRED IAW PIPER PA46-350P SERVICE MANUAL. OPERATIONAL CHECK FOUND SATISFACTORY. |  |  |  |
| C/W LYCOMING SERVICE BULLETIN 521 BY INSPECTION OF EXHAUST SYSTEM AND CROSSOVER PIPE. FOUND SATISFACTORY. |  |  |  |
| SOME SBS THAT APPLY TO THIS AIRCRAFT MAY NOT HAVE BEEN PERFORMED PER OWNER'S DECISION. C/W ALL APPLICABLE ADS THROUGHOUT THIS DATE. |  |  |  |