| N427AB | 2980 AIRWAY AVE, COSTA MESA, CA 92626 TEL: 714-433-2275, WWW.JAYSAIRCRAFT.COM FAA APPROVED REPAIR STATION # JVYR016L |  | DATE: 9/22/2020 |
| --- | --- | --- | --- |
| S/N: L-12384-61A |  |  | W.O.#: 23824 |
| MODEL: TIO-540-AE2A |  |  | TSMOH: |
| TACH TIME: |  |  | TOTAL TIME: 1616.2 |
| HOBBS TIME: 2248.9 |  |  |  |
| ENGINE LOG THE AIRCRAFT AND / OR COMPONENT IDENTIFIED ABOVE WAS REPAIRED AND INSPECTED IN ACCORDANCE WITH CURRENT FEDERAL AVIATION REGULATIONS AND WAS FOUND AIRWORTHY FOR RETURN TO SERVICE. PERTINENT DETAILS OF THE REPAIR / REPLACEMENT ARE ON FILE AT THIS AGENCY SIGNED: _____________________________________________________________________________ |  |  |  |
| C/W ANNUAL INSPECTION IAW PIPER PA46-350P MAINTENANCE MANUAL. |  |  |  |
| COMPRESSION VALUES: 1. 60/80, 2. 78/80, 3. 78/80, 4. 78/80, 5. 76/80, 6. 78/80 MASTER ORIFICE VALUE: 42 |  |  |  |
| SERVICED WITH 12 QTS OF AEROSHELL 15W-50 AND CH48103-1 OIL FILTER. AOA TAKEN. ENGINE WASHED, RUN UP AND INSPECTED. OPERATIONAL CHECK SATISFACTORY. |  |  |  |
| R&R INDUCTION AIR FILTER WITH NEW P/N P15-1936 IAW PIPER PA46-350P SERVICE MANUAL. OPERATIONAL AND LEAK CHECK FOUND SATISFACTORY. |  |  |  |
| MAGNETOS INSPECTED IAW PIPER PA46-350P, LYCOMING TIO-540, AND SLICK 6300 SERIES SERVICE MANUALS IAW ANNUAL INSPECTION GUIDELINES. RE-TIMED MAGNETOS AS REQUIRED. 500 HOUR MAGNETO INSPECTION DUE @ 2518.7 HOBBS HOURS, (269.8 HOBBS HOURS REMAINING.) |  |  |  |
| SPARK PLUGS ROTATED AND CLEANED, REPLACED GASKETS WITH NEW, P/N M674 (12) |  |  |  |
| C/W AD2015-19-07 LYCOMING FUEL LINE INSPECTION IAW PAR (e) (ii) AND LYCOMING SERVICE BULLETIN MSB 342G DATED 7/16/2003, SUPPLEMENT NO. 1 TO MSB 342G DATED 8/29/2013 AND SUPPLEMENT NO. 2 TO MSB 342G DATED 1/23/2014. FOUND SATISFACTORY, NEXT DUE 2358.9 HOBBS HOURS. |  |  |  |
| SOME SBS THAT APPLY TO THIS AIRCRAFT MAY NOT HAVE BEEN PERFORMED PER OWNER'S DECISION. C/W ALL APPLICABLE ADS THROUGHOUT THIS DATE. |  |  |  |