| N427AB | 2980 AIRWAY AVE, COSTA MESA, CA 92626 TEL: 714-433-2275, WWW.JAYSAIRCRAFT.COM FAA APPROVED REPAIR STATION # JVYR016L |  | DATE: 11/23/2020 |
| --- | --- | --- | --- |
| S/N: L-12384-61A |  |  | W.O.#: 23976 |
| MODEL: TIO-540-AE2A |  |  | TSMOH: |
| TACH TIME: |  |  | TOTAL TIME: 1658.2 |
| HOBBS TIME: 2290.9 |  |  |  |
| ENGINE LOG THE AIRCRAFT AND / OR COMPONENT IDENTIFIED ABOVE WAS REPAIRED AND INSPECTED IN ACCORDANCE WITH CURRENT FEDERAL AVIATION REGULATIONS AND WAS FOUND AIRWORTHY FOR RETURN TO SERVICE. PERTINENT DETAILS OF THE REPAIR / REPLACEMENT ARE ON FILE AT THIS AGENCY SIGNED: _____________________________________________________________________________ |  |  |  |
| SERVICED WITH 12 QTS OF AEROSHELL 15-W50 AND CH4103-1 OIL FILTER. AOA TAKEN. ENGINE WASHED, RUN UP AND INSPECTED. OPERATIONAL CHECK SATISFACTORY. |  |  |  |