| N427AB | 2980 AIRWAY AVE, COSTA MESA, CA 92626 TEL: 714-433-2275, WWW.JAYSAIRCRAFT.COM FAA APPROVED REPAIR STATION # JVYR016L |  | DATE: 9/23/2020 |
| --- | --- | --- | --- |
| S/N: 4636324 |  |  | W.O.#: 23824 |
| MODEL: PA46-350P |  |  | TSMOH: |
| TACH TIME: |  |  | TOTAL TIME: 2248.9 |
| HOBBS TIME: 2248.9 |  |  |  |
| AIRFRAME LOG THE AIRCRAFT AND / OR COMPONENT IDENTIFIED ABOVE WAS REPAIRED AND INSPECTED IN ACCORDANCE WITH CURRENT FEDERAL AVIATION REGULATIONS AND WAS FOUND AIRWORTHY FOR RETURN TO SERVICE. PERTINENT DETAILS OF THE REPAIR / REPLACEMENT ARE ON FILE AT THIS AGENCY SIGNED: _____________________________________________________________________________ |  |  |  |
| C/W ANNUAL INSPECTION IAW PIPER PA46-350P MAINTENANCE MANUAL. |  |  |  |
| FAR 91-207(d) COMPLY WITH YEARLY INSPECTION OF ELT BATTERY BY PHYSICAL TEST AND VISUAL INSPECTION. OPERATION CHECKED SATISFACTORY. ELT BATTERY EXPIRES 8/2021 |  |  |  |
| PITOT STATIC TRANSPONDER TEST DUE DATE: 8/2021 |  |  |  |
| VACUUM SYSTEM INSPECTED IAW PIPER PA46-350P SERVICE MANUAL IAW ANNUAL INSPECTION GUIDELINES. R&R VACUUM REGULATOR FILTER WITH NEW P/N B3-5-1. CENTRAL INSTRUMENT AND VACUUM PUMP FILTER INSPECTED AND FOUND SATISFACTORY. |  |  |  |
| R&R FUEL BOWL AFTER INSPECTED IAW PIPER PA46-350P SERVICE MANUAL. R&R O-RINGS WITH NEW P/N MS29513-007, MS29513-04, AND MS29513-232. OPERATIONAL AND LEAK CHECK FOUND SATISFACTORY. |  |  |  |
| R&R NOSE TIRE AND INNER TUBE WITH NEW GOODYEAR FLIGHT CUSTOM III 5.00X5 6 PLY P/N 505C66-5 AND INNER TUBE WITH NEW MICHELIN AIR STOP 5.00X5 P/N 097-908-0 IAW PIPER PA46-350P AND CLEVELAND WHEEL AND BRAKE SERVICE MANUALS. OPERATIONAL AND LEAK CHECK FOUND SATISFACTORY. |  |  |  |
| SOME SBS THAT APPLY TO THIS AIRCRAFT MAY NOT HAVE BEEN PERFORMED PER OWNER'S DECISION. C/W ALL APPLICABLE ADS THROUGHOUT THIS DATE. |  |  |  |