| N427AB | 2980 AIRWAY AVE, COSTA MESA, CA 92626 TEL: 714-433-2275, WWW.JAYSAIRCRAFT.COM FAA APPROVED REPAIR STATION # JVYR016L |  | DATE: 6/3/2020 |
| --- | --- | --- | --- |
| S/N: 4636324 |  |  | W.O.#: 23695 |
| MODEL: PA46-350P |  |  | TSMOH: |
| TACH TIME: |  |  | TOTAL TIME: 2156.8 |
| HOBBS TIME: 2156.8 |  |  |  |
| AIRFRAME LOG THE AIRCRAFT AND / OR COMPONENT IDENTIFIED ABOVE WAS REPAIRED AND INSPECTED IN ACCORDANCE WITH CURRENT FEDERAL AVIATION REGULATIONS AND WAS FOUND AIRWORTHY FOR RETURN TO SERVICE. PERTINENT DETAILS OF THE REPAIR / REPLACEMENT ARE ON FILE AT THIS AGENCY SIGNED: _____________________________________________________________________________ |  |  |  |
| TROUBLESHOT CABIN PRESSURIZATION IAW PIPER PA46-350P SERVICE MANUAL. FOUND ALL CABIN ORIFICES SEALED AS REQUIRED. PRESSURIZATION TEST FOUND LEAK ON R/H SIDE INTERCOOLER CLAMP. R&R R/H SIDE INTERCOOLER CLAMP WITH NEW IAW LYCOMING TIO-540 AND PIPER PA46-350P SERVICE MANUAL. OPERATIONAL AND LEAK CHECK FOUND SATISFACTORY. |  |  |  |