| N427AB | 2980 AIRWAY AVE, COSTA MESA, CA 92626 TEL: 714-433-2275, WWW.JAYSAIRCRAFT.COM FAA APPROVED REPAIR STATION # JVYR016L |  | DATE: 11/26/2019 |
| --- | --- | --- | --- |
| S/N: L-12384-61A |  |  | W.O.#: 23448 |
| MODEL: TIO-540-AE2A |  |  | TSMOH: |
| TACH TIME: |  |  | TOTAL TIME: 1492.7 |
| HOBBS TIME: 2125.4 |  |  |  |
| ENGINE LOG THE AIRCRAFT AND / OR COMPONENT IDENTIFIED ABOVE WAS REPAIRED AND INSPECTED IN ACCORDANCE WITH CURRENT FEDERAL AVIATION REGULATIONS AND WAS FOUND AIRWORTHY FOR RETURN TO SERVICE. PERTINENT DETAILS OF THE REPAIR / REPLACEMENT ARE ON FILE AT THIS AGENCY SIGNED: _____________________________________________________________________________ |  |  |  |
| SERVICED WITH 12 QTS OF AEROSHELL W100 AND CH48103-1 OIL FILTER. AOA TAKEN. SERVICED WITH LYCOMING ADAPTIVE P/N LW-16702. ENGINE WASHED, RUN UP AND INSPECTED. OPERATIONAL CHECK SATISFACTORY. |  |  |  |
| R&R SPARK PLUGS WITH NEW P/N URHB36S (12) IAW LYCOMING TIO-540 AND PIPER PA46-350P SERVICE MANUALS. OPERATIONAL CHECK FOUND SATISFACTORY. |  |  |  |